PlatformIO + VS Code

Beginner's Guide for Arduino Uno

# 1. What is PlatformIO?

PlatformIO is an extension for VS Code that lets you write, compile, and upload Arduino code — just like the Arduino IDE, but more powerful and organized.

💡 Think of it as a better version of the Arduino IDE that lives inside VS Code.

# 2. Installing PlatformIO in VS Code

### Step 1 — Open VS Code and go to Extensions

Click the square icon on the left sidebar (or press Ctrl+Shift+X)

### Step 2 — Search and Install

Search: PlatformIO IDE

Click Install

Wait for it to finish (may take a few minutes)

Restart VS Code when prompted

### Step 3 — Open PlatformIO Home

Click the ant icon 🐜 on the left sidebar

Click 'Open' to go to PlatformIO Home

💡 The ant icon is the PlatformIO logo. If you don't see it, restart VS Code.

# 3. Creating Your First Project

### Step 1 — Create New Project

PlatformIO Home → New Project

Name: MyFirstProject

Board: Arduino Uno

Framework: Arduino

Click Finish

### Step 2 — Your Project Structure

MyFirstProject/

├── src/

│   └── main.cpp     ← Write your code here!

├── lib/             ← Put libraries here

├── include/         ← Header files

└── platformio.ini   ← Project settings

⚠️  Always write your code inside src/main.cpp — not anywhere else!

### Step 3 — Basic Code Template

#include <Arduino.h>    // Always include this in PlatformIO!

void setup() {

pinMode(13, OUTPUT);  // Setup runs once

}

void loop() {

digitalWrite(13, HIGH); // Loop runs forever

delay(1000);

digitalWrite(13, LOW);

delay(1000);

}

💡 Unlike Arduino IDE, PlatformIO requires #include <Arduino.h> at the top of every file!

# 4. Building and Uploading Code

| Action | Button | Keyboard Shortcut |
| --- | --- | --- |
| Build/Compile code | ✔ Checkmark at bottom bar | Ctrl+Alt+B |
| Upload to Arduino | → Arrow at bottom bar | Ctrl+Alt+U |
| Open Serial Monitor | 🔌 Plug icon at bottom bar | Ctrl+Alt+S |
| Clean build files | 🗑 Trash icon | - |

💡 Look at the bottom blue status bar in VS Code — that's where all PlatformIO buttons are!

# 5. Managing Multiple Projects

You have two ways to handle multiple Arduino projects:

## Option A — One Folder, Multiple Environments (Related Projects)

Use this when your projects are similar or share code (e.g., testing relay on Uno and Nano).

### platformio.ini setup:

[env:relay_project]

platform = atmelavr

board = uno

framework = arduino

src_filter = -<*> +<relay.cpp>   ; Only compile relay.cpp

[env:main_project]

platform = atmelavr

board = uno

framework = arduino

src_filter = -<*> +<main.cpp>    ; Only compile main.cpp

### Switching between environments:

Bottom blue bar → click environment name → choose which one to build/upload

## Option B — Separate Folders (Recommended for Different Projects)

Use this when projects are completely different (e.g., relay project vs sensor project).

### Folder structure:

ARDUINO_PROJECTS/

├── RELAY_PROJECT/

│   ├── platformio.ini

│   └── src/main.cpp

├── SENSOR_PROJECT/

│   ├── platformio.ini

│   └── src/main.cpp

└── AI_CAMERA_PROJECT/

├── platformio.ini

└── src/main.cpp

### Opening a project in VS Code:

File → Open Folder → choose your specific project folder

Each project opens independently with its own settings

💡 For your thesis, use Option B — keep AI camera, relay, and other modules as separate folders.

# 6. Useful Terminal Commands

You can also type commands in the VS Code terminal (Ctrl+`) instead of clicking buttons:

| Command | What it does |
| --- | --- |
| pio run | Build all environments |
| pio run -e relay_project | Build only relay_project environment |
| pio run -e relay_project --target upload | Upload relay_project to Arduino |
| pio device monitor | Open serial monitor |
| pio device list | Show connected USB devices |
| pio run --target clean | Delete build files (fixes weird errors) |
| pio lib install "LibraryName" | Install a library |

# 7. Installing Libraries

### Method 1 — PlatformIO Home (Easiest)

PlatformIO Home → Libraries → search library name → Add to Project

### Method 2 — Terminal

pio lib install "LibraryName"

Examples:

pio lib install "Servo"

pio lib install "RBDDimmer"

### Method 3 — platformio.ini (Recommended)

[env:uno]

platform = atmelavr

board = uno

framework = arduino

lib_deps =

Servo

RBDDimmer

💡 Method 3 is best for thesis — libraries automatically install when you build, even on a new PC.

# 8. Common Errors and Fixes

| Error | Cause | Fix |
| --- | --- | --- |
| 'Arduino.h' not found | Missing #include | Add #include <Arduino.h> at top of file |
| No such file or directory | Wrong file path | Check your src_filter in platformio.ini |
| Upload failed / port error | Wrong COM port or driver | Check Device Manager, reinstall CH340 driver |
| Build succeeds but nothing happens | Wrong environment selected | Check bottom bar — select correct environment |
| Weird errors after editing | Corrupted build cache | Run: pio run --target clean, then rebuild |

# 9. Quick Reference — Board Names

| Board | platformio.ini board value |
| --- | --- |
| Arduino Uno | uno |
| Arduino Nano | nanoatmega328 |
| Arduino Mega | megaatmega2560 |
| Arduino Leonardo | leonardo |
| ESP32 | esp32dev |
| NodeMCU ESP8266 | nodemcuv2 |

Made for Arduino + PlatformIO beginners 🎓